PCB CHARACTERIZATION

AccuCAD

14905 E. Gilbert, Wichita, KS 67230

E-Mail lau7@cox.net

Ph:  316-733-1689

Date: _23APR03_					Designer:  Margie Lillig

Part Number: ____RX-002______ Rev.: _2_ 	PCB Name: ___Receiver_____________

No. of Layers: ____2____	Board Thickness: __.062+/-.004_____

Image Width___2.220_____	Image Length: __1.250_____

Panelize best way to fit panel

Material: _*_ FR4 Natural   ___ Teflon (Teflon Type: ______________________)

Copper Thickness: __1___OZ._ALL_LAYERS_____________________________________

Etch Tolerance: _+/- .002_ 	Special Features/Tolerance: _________________________

Smallest Supplied Space: _. 008_		Smallest Supplied Trace Width: _. 008_

No. of Holes: _53___	 Minimum Hole Size: _. 012_ Tolerance: __+. 000, -.010____

No. of tools: __4__

Surface Mount: _*_ Yes ___ No		No. of Surface Mount Pads: _72___

Sides Surface Mount: ___ 1 _*_ 2           Continuity Test:_*_ Yes ___ No

Soldermask: _*_ Liquid Photoimageable ___ Screened ___ None

Soldermask on: _*_ Both sides ___ Top side only ___ Bottom side only

Process: ____*____ Sn/Pb HASL

_________ Immersion Gold

__________________________________________________________________________

Gold Edge: __ Yes _*__ No

Selective Gold: ___ Yes _*_ No		 Gold Square Inches: ___ Gold Thickness: ______

Controlled Impedance: ___ Yes _*_ No	Silk Screen: ___None Side: _*_ T __ B

Special Instructions: _____________________________________________________

___________________________________________________________________________

Zip File Contents:

Gerber files are in RS-274-X

Total # of Files	12

Image

(POS) (NEG)	File Name		Description

P		TOP-RX002.PHO		Layer 1 (Component Side)

P		BOT-RX002.PHO		Layer 6	(Solder Side - Bottom)

P		TMK-RX002.PHO		Top Side Solder Mask

P		BMK-RX002.PHO		Bottom Side Solder Mask

P		TPS-RX002.PHO			Top Side Paste Stencil

P		TSL-RX002.PHO		Top Side Silkscreen

P		TAS-RX002.PHO		Top Side Assembly

P		FAB-RX002.PHO		Drill Drawing

PTH-RX002.DRL		Plated thru Drill file

NPT-RX002.DRL		Not plated drill file

PNP-RX002.ASC		Manufacturing Pick & Place Data

ReadmeRX002.TXT		This document