PCB CHARACTERIZATION

AccuCAD

14905 E. Gilbert, Wichita, KS 67230

E-Mail lau7@cox.net

Ph:  316-733-1689

Date: _21AUG03_						Designer Margie Lillig

Part Number: ____TX-002______ 	Rev.: _3_ PCB Name: ___Transmitter SM_______

No. of Layers: ____2____		Board Thickness: __.062+/-.004_____

Image Width___ .990 _____		Image Length: __5.230_____

Panelize best way to fit panel

Material: _*_ FR4 Natural   ___ Teflon (Teflon Type: ______________________)

Copper Thickness: __1___OZ._ALL_LAYERS_____________________________________

Etch Tolerance: _+/- .002_	Special Features/Tolerance: _________________________

Smallest Supplied Space: _. 008_		Smallest Supplied Trace Width: _. 008_

No. of Holes: _20___  Minimum Hole Size: _.012_ Tolerance:__+.000,-.010____

No. of tools:__1__

Surface Mount _*_ Yes ___ No		No. of Surface Mount Pads: _56___

Sides Surface Mount: ___ 1 _*_ 2           Continuity Test:_*_ Yes ___ No

Soldermask: _*_ Liquid Photoimageable ___ Screened ___ None

Soldermask on: _*_ Both sides ___ Top side only ___ Bottom side only

Process: ____*___Sn/Pb HASL

________Immersion Gold

Gold Edge: __ Yes _*__ No

Selective Gold: ___ Yes _*_ No Gold Square Inches: ___ Gold Thickness: ______

Controlled Impedance: ___ Yes _*_ No    Silk Screen: ___None Side: _*_ T _*_ B

Special Instructions: _____________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Zip File Contents:

Gerber files are in RS-274-X

Total # of Files	13

Image

(POS) (NEG)	File Name			Description

P		TOP-TX-002.PHO		Layer 1 (Component Side)

P		BOT-TX-002.PHO		Layer 2	(Solder Side - Bottom)

P		TMK-TX-002.PHO		Top Side Solder Mask

P		BMK-TX-002.PHO		Bottom Side Solder Mask

P		TPS-TX-002.PHO		Top Side Paste Stencil

P		BPS-TX-002.PHO		Bottom Side Paste Stencil

P		TSL-TX-002.PHO		Top Side Silkscreen

P		BSL-TX-002.PHO		Bottom Side Silkscreen

P		TAS-TX-002.PHO		Top Side Assembly

P		FAB-TX-002.PHO		Drill Drawing

PTH-TX-002.DRL		Plated thru Drill file

PNP-TX-002-0603.ASC		Manufacturing Pick & Place Data

ReadmeTX-002-0603.TXT	This document